- Purpose : To ensure safe handling of overhead cranes to prevent any mishaps and injuries.

- Scope : Applicable to all crane operators in the organization

- Method:

| Activity | Activity Description | Responsibility |
| --- | --- | --- |
| Crane Operation | Only authorized person should operate the crane. | Supervisor |
|  | Before start of shift take trial forward, backward movements of the crane. | Crane operator |
|  | Before start of shift check the condition of pendant switch. |  |
|  | Check chain, slings, hooks before operation. |  |
|  | Check the pallet condition before lifting. |  |
|  | Use two chains for loading/ unloading of steel bars, so that material is in balance condition |  |
|  | If the material is loaded in bin then it should not be loaded above the bin level. | Helper |
|  | Do not lift the material more than crane & chain capacity. | Crane operator |
|  | Material should be moved from marked gang ways only. |  |
|  | While moving the material, care should be taken that no person is standing in the gang ways. |  |
|  | The material should not be lifted one foot above the ground level. |  |
|  | If power failure occurs while handling the crane, nobody should be near the hanging load. | Supervisor |
|  | If the hoist vertical movement is failed & if chain pulley block is required to use, then proper care to be taken while clamping the chain pulley block, also check the capacity. |  |
|  | Periodic record of testing of crane, chain, slings, ropes should be maintained. | Maintenance |
|  | Crane maintenance record should be maintained. | Maintenance |
|  | Any problem/ breakdown should be informed to Maintenance department. | Crane operator |
|  | After operation crane should be parked at proper position. |  |
|  | During idle condition the crane hook should be above the working level. |  |

- Safety Practices:

- The operator should always wear Helmet, Shoes, and hand gloves while operating the crane.

- Monitoring:

The Supervisor is overall responsible for monitoring that only authorized person is operating the crane.

- Records:

- List of authorized crane operators

- Records of testing of Crane, hoist, ropes, chains, etc.

- Maintenance records of crane

| PREPARED BY. | APPROVED BY. |
| --- | --- |
| Sign and Name initials | Sign and Name initials |