|  | SAFE WORKING PROCEDURE(SWP) Form No. QMS-F-011 Rev. 00 |  |  |
| --- | --- | --- | --- |
|  | ACTIVITY: ANGULAR GRINDER |  |  |
|  | Document No. | Department | Integration |
|  |  | SWP No. | 004 |
|  | Issue No. & Date | Revision No. |  |
|  |  | Date |  |

| Brief Description of Activity:ANGULAR GRINDER |  |  |
| --- | --- | --- |
| Personal Protective Equipment or other required equipment or other safety considerations:-: Risk and Hazards Prevention: -Entanglement – No loose Clothing, No Jewellery or accessories, Long Hair must be contained.  Use extraction system or dust mask to avoid inhalation of fumes and dusts. Burns from hot material or abrasion after/during grinding. DO NOT use this power tool unless you have received instruction in its safe use and operation and have permission. |  |  |
| Work Instruction: Step 1 Step 2 Keep fingers, hands and cables free of grinding disc  Check that the disk runs ‘true’ and does not wobble. Excess vibration must be identified and addressed immediately Do not make adjustments while wheel is running.  ; Un-plug grinder from wall/socket before changing the disk or making adjustments.  Ensure guarding is in place before re-starting grinder  Allow grinding wheel to reach working speed before applying to work piece. Increase working loads gradually. Never abruptly. Use smooth fluid motions Be aware of flying sparks and particulate... Step 3; Step 4. Aim debris away from self and other people as well as flammable materials. Turn grinder off when not in use; Ensure machines have been isolated from power sources before being cleaned, adjusted, maintained or repaired.  En-plug when not in use. Ensure Grinding wheel has having more RPM than Machine or Grinder RPM. Grinder should have handle and guard. |
|  | Checked by | Approved by |
| Signature |  |  |
| Date |  |  |