| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 10 |  |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |
| Process Name/ Operation Description: Inward Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  |  | Approved by :- |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility | Reaction Plan |
|  | No. | Product | Process |  |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |
|  |  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  |
| Raw Material Inspection | 1 | Raw material Grade | - |  | - | EN8M |  | Supplier T.C. | One T.C. | /Lot |  | Third party test/3 month | Inword Q.C. Insp... | If Reject Sent Back to Vendor |
|  | 2 | O. Dia. | - |  | - | Ø 40.5±0.1 |  | Micrometer | As per sampling Plan |  |  | Supplier TC |  |  |
|  | 3 | Appearance | - |  | - | No pit marks, Rusty |  | Visual | As per sampling Plan |  |  | In word insp. Report |  |  |
|  | 4 | Color Code | - |  | - | Green |  | Visual | 100% | /Per Lot |  |  |  |  |

Note:-

Vendor PDI – Vendor pre-despatch inspection report

SSP(N) – Single Sampling Plan (Normal)

I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 20 |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |  |
| Process Name/ Operation Description: Parting & Drilling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production |
| TRAUB A-42 | 1 | Total length | - | - | 49.0±0.15 |  | Snap Gauge | 1Pc | /10Pcs |  |  | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  | HIPR (Record to be maintain of min-max value of 1 part) | Q.C. INSP. |  |  |
|  | 2 | O.Dia | - | - | Ø 40.5±0.1 |  | Micrometer | 1pc | /Sett |  |  | Q.C. INSP. |  |  |
|  | 3 | O.Chamfer | - | - | 0.5x45˚ (Both sides) |  | Profile Projector | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |  |
|  | 4 | I.D. | - | - | Ø8.55+0.15 |  | Plain Plug Guge | 1Pc | /10Pcs |  |  | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |  |
|  | 5 | Depth | - | - | 25±0.5 |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |  |
|  | 6 | App. | - | - | No drill out Dent & damages |  | Visual | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |  |
|  | 7 | - | Parting insert |  | 3.0 mm |  | Visual | At the time of setting | Start the Shift |  | - | - |  |  |
|  | 8 | - | Drill |  | 8.5 mm |  | Visual |  |  |  |  |  |  |  |
|  | 9 | - | R.P.M. |  | 500 ~ 600 |  | Machine Chart |  |  |  |  |  |  |  |
|  | 10 | - | FEED |  | 60 |  | Machine Chart |  |  |  |  |  |  |  |
|  | 11 | - | Cutting Oil | - | Vorstab |  | Visual |  |  |  |  |  |  |  |
|  | 12 | - | Coolent Concentration | - | 4% to 6% |  | Refractometer |  |  |  |  |  |  | - |

Note: - HIPR – Hourly Inspection & Production Report  I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 30 |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |  |
| Process Name/ Operation Description: Milling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production |
| Milling M/C | 1 | Dim | - | - | 43.5±0.1 |  | Length gauge | 1Pc | /10Pcs |  | - | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  | HIPR (Record to be maintain of min-max value of 1 part) | Q.C. INSP. |  |  |
|  | 2 | A/F | - | - | 28.0-0.2 |  | Snap Gauge | 1Pc | /10Pcs |  |  | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |  |
|  | 7 | - | Cutter | - | Side and Face cutter |  | Visual | At the time of setting | Start the Shift |  | - | - |  |  |
|  | 9 | - | R.P.M. | - | 500 ~ 800 |  | - |  |  |  |  |  |  |  |
|  | 10 | - | FEED | - | Manual |  | - |  |  |  |  |  |  |  |
|  | 11 | - | Tool Resharpning | - | 1000 no./Every Resharp |  | Visual | At the time of setting | Start the Shift |  | - | - |  |  |
|  | 12 | - | Cutting Oil | - | Vorstab |  | Visual |  |  |  |  |  |  |  |
|  | 13 | - | Coolent Concentration | - | 4% to 6% |  | Refractometer |  |  |  |  |  |  | - |

Note: - HIPR – Hourly Inspection & Production Report I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 40 |  |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |
| Process Name/ Operation Description: CNC Turning 1st |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  |  | Approved by :- |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility | Reaction Plan |
| CNC LATHE M/C | No. | Product | Process |  |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |
|  |  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  |
|  | 1 | Appearance | - |  | - | No Chamfer Out |  | Visual | 5Pcs | /2Hrs |  | HIPR (Record to be maintain of min-max value of 1 part) | Q.C. INSP. | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production |
|  | 2 | Total Length | - |  | - | 48.5±0.1 |  | Length Gauge | 1Pc | /10Pcs |  |  | OPERATOR |  |
|  |  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |
|  | 3 | Dia | - |  | - | 40.5±0.2 |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |
|  | 4 | Dim | - |  | - | 7.0 |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |
|  | 5 | Chamfer | - |  | - | 1x45˚ |  | Profile Projector | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |
|  | 6 | - | Insert |  |  | TNMG 0.8 |  | Visual | At the time of setting | Start the Shift |  | - | - |  |
|  | 7 | - | RPM |  |  | 2000-2200 |  | Machine Chart |  |  |  |  |  |  |
|  | 8 | - | Feed |  |  | 0.12-0.15 |  | Feed Chart |  |  |  |  |  |  |
|  | 9 | - | Cutting Oil |  |  | Vorstab |  | Visual |  |  |  |  |  |  |

Note: - HIPR – Hourly Inspection & Production Report I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 50 |  |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |
| Process Name/ Operation Description: DEBURRING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  |  | Approved by :- |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility | Reaction Plan |
| BELT GRINDER M/C | No. | Product | Process |  |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |
|  |  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  |
|  | 1 | Appearance | - |  | - | No burr Allowed, No dent damages |  | Visual | 5Pcs | /2Hrs |  | HIPR | Q.C. INSP. | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production |
|  | 2 | - | Belt |  |  | 1220x150 (60 grade) |  | Visual | At the time of setting | Start the Shift |  | - | - |  |
|  | 3 | - | RPM |  |  | 300-500 |  | Machine Chart |  |  |  |  |  |  |
|  | 4 | - | Feed |  |  | Manual |  | Feed Chart |  |  |  |  |  |  |
|  | 5 | - | Cutting Oil |  |  | Vorstab |  | Visual |  |  |  |  |  |  |

Note: - HIPR – Hourly Inspection & Production Report I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 60 |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |
| Process Name/ Operation Description: CNC Turning 2nd |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  | Control Method |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production |
| CNC LATHE M/C | 1 | Appearance | - | - | No Tool mark, dent, Damage, Etc. |  | Visual | 100% | Lot | No record | Operator |  |  |
|  | 2 | T. Length | - | - | 48.0±0.2 |  | Vernier Caliper | 5Pcs | /2Hrs | HIPR (Record to be maintain of min-max value of 1 part) | Q.C. INSP. |  |  |
|  | 3 | Dia | - | - | 38.2-0..2 |  | Snap Gauge | 1Pc | /10Pcs |  | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 4 | G.Dia | - | I | 33+0.2 |  | Snap gauge | 1Pc | /10Pcs |  | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 5 | Dim | - | - | 7.0 |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 6 | Dim | - | - | 5 |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 7 | Dim | - | I | 4.5+0.2 |  | Width Gauge | 1Pc | /10Pcs |  | OPERATOR |  |  |
|  |  |  |  |  |  |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 8 | Dim | - | - | 9.0 |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 9 | Thread Lenght | - | - | 22.5 |  | Profile Projector | 5Pcs | /2Hrs | HIPR (Record to be maintain of min-max value of 1 part) | Q.C. INSP. |  |  |
|  | 10 | G.Dia | - | I | 33+0.2 |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 11 | Dim | - | I | 4.5+0.2 |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 12 | Thread | - | - | M38x1-6g |  | Thread Ring Gauge | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 13 | Chamfer | - | - | 1x30° |  | Profile Projector | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 14 | Radius | - | - | R0.5 |  | Profile Projector | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 15 | Dia | - | - | 38.0-0.2 |  | Vernier Caliper | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 16 | Radius | - | - | R0.5 |  | Profile Projector | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 17 | Concentricity | - | - | 0.1 w.r.t.P |  | V-Block with Dial (Before Threading) | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 18 | Concentricity | - | - | 0.1 w.r.t.P |  | V-Block with Dial (Before Threading) | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 19 | Chamfer | - | - | 1x45° |  | Profile Projector | 5Pcs | /2Hrs |  | Q.C. INSP. |  |  |
|  | 20 | - | Insert | - | TNMG 0.4 |  | Visual | At The Time Of Setting | At The Time Of Shift Start | - | - |  |  |
|  | 21 |  | Insert | - | 3mm Grooving |  | Visual |  |  |  |  |  |  |
|  | 22 | - | Insert | - | ER1.5 Threading |  | Visual |  |  |  |  |  |  |
|  | 23 | - | RPM | - | 2000-2200 |  | Machine Chart |  |  |  |  |  |  |
|  | 24 | - | Feed | - | 0.12-0.15 |  | Machine Chart |  |  |  |  |  |  |
|  | 25 | - | Cutting Oil | - | Vorstab |  | Visual |  |  |  |  |  |  |
|  | 26 | - | Coolant Concentration | - | 4% To 6% |  | Refracto Meter |  |  |  |  |  |  |

Note: - HIPR – Hourly Inspection & Production Report I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 70 |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |  |
| Process Name/ Operation Description: FINISH DRILLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production - |
| Drill M/C | 1 | I.Dia | - | - | 8.55+0.1 |  | Vernier Caliper | 5Pcs | /2Hrs |  | HIPR (Record to be maintain of min-max value of 1 part) | Q.C. INSP. |  |  |
|  | 2 | Drill Depth | - | - | 25.0+0.5 |  | Vernier Caliper | 5Pcs | /2Hrs |  |  | Q.C. INSP. |  |  |
|  | 3 | - | Drill |  | 8.5 |  | Visual | At the time of setting | Start the Shift |  | - | - |  |  |
|  | 4 | - | R.P.M. |  | 500 ~ 600 |  | Machine Chart |  |  |  |  |  |  |  |
|  | 5 | - | FEED |  | Manual |  | - |  |  |  |  |  |  |  |
|  | 6 | - | Cutting Oil | - | Vorstab |  | Visual |  |  |  |  |  |  |  |
|  | 7 | - | Coolent Concentration | - | 4% to 6% |  | Refractometer |  |  |  |  |  |  |  |

Note: - HIPR – Hourly Inspection & Production Report I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 80 |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |  |
| Process Name/ Operation Description: Taping (M10x1.5-6H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |  |
| Machine, Device, Mg Books for Mfg. | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  | Stop the Process, Quarantine the defects if any & If not ok , inform to supervisor, correct it , Re-verify five jobs & If OK then start production - |
| Taping M/C | 1 | Taping | - | I | M10x1.5 – 6g |  | Thread Plug Gauge | 1Pc | /10Pcs |  | - | OPERATOR |  |  |
|  |  |  |  |  |  |  | Thread Plug Gauge | 5Pcs | /2Hrs |  | HIPR | Q.C. INSP. |  |  |
|  | 2 | Taping Depth | - | I | 20 min |  | TPG+ Vernier | 5Pcs | /2Hrs |  | HIPR | Q.C. INSP. |  |  |
|  | 3 | - | Tap |  | M10x1.5 – 6g |  | Visual | At the time of setting | Start the Shift |  | - | - |  |  |
|  | 4 | - | R.P.M. |  | 500 ~ 600 |  | Machine Chart |  |  |  |  |  |  |  |
|  | 5 | - | FEED |  | Manual |  | - |  |  |  |  |  |  |  |
|  | 6 | - | Cutting Oil | - | Vorstab |  | Visual |  |  |  |  |  |  |  |
|  | 7 | - | Coolent Concentration | - | 4% to 6% |  | Refractometer |  |  |  |  |  |  |  |

Note: - HIPR – Hourly Inspection & Production Report I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  | Date (Drg) : 13.03.21 |  |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  | Customer Approval :- |  |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 90 |  |  |  |  |  | Prepared by :- |  |  |  | Customer Quality Approval :- |  |  |  |  |
| Process Name/ Operation Description: Plating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |  |
| PLATING (In-word Inspection) | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  |  | Control Method |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  | If not ok , inform to Supplier and send to back |
|  | 1 | Surface Finish | - | - | FeZn8-Y (Zinc Plating +Yellow pass. Lacqering ) |  | Visual | As per sampling Plan |  |  | In-word insp. report | Q.C Insp. |  |  |
|  | 2 | Plating Thick. | - | - | 8µ~ 12µ |  | Supplier T.C | 1T.C | /Lot |  |  |  |  |  |
|  | 3 | Appearance | - | - | No Dents, Poor Plating, Rusty etc. |  | Visual | As per sampling Plan |  |  |  |  |  |  |

Note: - HIPR – Hourly Inspection & Production Report,  I – Important Dimension, P-Past Problem.

| X Prototype X Pre-launch  Production Control Plan Number : CP06 |  |  |  |  |  | Key Contact/Phone : 1) Mr. Khandlwal U R 2) Mr.Kurund M A 9552556659 |  |  |  |  | Date (Drg) : 13.03.21 |  | Date (Rev.): |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number/Latest Change Level: F2DZ04603O/XH |  |  |  |  |  | Core Team: 1) Mr. Pankaj Khadase 2) Mr. Prashant Kamble 3) Mr. Dyaneshwar Ingole 4) Mr Mohan Jadhav |  |  |  |  | Customer Approval :- |  |  |  |
| Part Name/Description : FORK BOLT J1A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Process Number : 100 |  |  |  |  |  | Prepared by :- |  |  |  |  | Customer Quality Approval :- |  |  |  |
| Process Name/ Operation Description: Final Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier/Plant Sharp Engineers A-31, MIDC, Waluj, Aurangabad. |  |  |  | Supplier Code :-100106 |  | Approved by :- |  |  |  |  |  |  |  |  |
| Final Inspection | Characteristics |  |  | Special Char. Class | Methods |  |  |  |  |  |  | Responsibility |  | Reaction Plan |
|  | No. | Product | Process |  | Product/ Process Specification/ Tolerance |  | Evaluation/ Measurement Technique | Sample |  | Control Method |  |  |  |  |
|  |  |  |  |  |  |  |  | Size | Freq. |  |  |  |  | Rejection Hand-over to Final inspection in-charge, If observed major rejection inform to F.Supervisor |
|  | 1 | Thread | - | - | M10x1.5-6H |  | T.P.G. | As per sampling Plan |  | Inspection Checksheet |  | Q.C Insp. |  |  |
|  | 2 | Taping Depth | - | I | 20 min |  | Matching Bolt | 100% | /Lot |  |  |  |  |  |
|  | 3 | Appearance | - | - | No Dents, Poor Plating, Rusty etc. |  | Visual | 100% | /Lot |  |  |  |  |  |

| PDI (110) | 1 | Product Parameter | -- | -- | All parameter should be as per standard | As per Std. | As per Sampling Plan | - | PDIR | Q.C Insp. | If observed NG re-check 100% and in-form to concerned person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Packing & Forwarding (120) | 1 | Packing & Forwarding | - | - | As per Packing Standard & Forwarding as per dispatch plan | Visual | 100% | /Lot | Verification at out-word & Invoicing | Packer & Security guard | Re-Packing and re-counting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note: - HIPR – Hourly Inspection & Production Report,  I – Important Dimension, P-Past Problem.